RFID Development Documentation

V2.9

Table of contents

1. Common method usage process	3

1.1 initialization	3

1.2 Inventory	4

1.3 read tag	4

1.4 write tag	4

1.5 get the power of output	5

1.6 set the power of output	5

1.7 Parameter setting	5

1.8 Long range (with handle) device control scanning head light	7

2. Interface Callback	8

note：there are some parameters do not work,they have a default value,it is necessary to focus on them in this version。	9

onInventoryTag	9

onInventoryTagEnd	9

3. RFID api	9

3.1 init【initialization】	9

3.2 release【release resources】	10

3.3 disConnect【disconnect rfid module】	10

3.4 isConnected【RFID connection status 】	10

3.5 registerCallback【register CallBack】	11

3.6  startInventory【User define session and target inventory】	11

3.7 stopInventory【stop tag inventory】	11

3.8 readTag【read tag】	12

3.9 readDataByTid【Tag data is read via TID】	12

3.10 inventorySingle【single tag query】	13

3.11 writeTag【Write Tag】	13

3.12 writeTagByTid【Write data to each storage area through tid】	14

3.13 writeEpc【write epc number】	14

3.14 killTag【Kill Tag】	15

3.15 KillbyTID【The tag is destroyed by TID】	15

3.16 lockTag【Lock Tag】	16

3.17 LockbyTID【The tag is locked using the TID】	17

3.18 setQueryMode【config query model】	18

3.19 getQueryMode【get query mode】	19

3.20 getFirmwareVersion【Get Reader Firmware Version.】	19

3.21 setOutputPower【Set output power】	19

3.22 getOutputPower【Query output power】	20

3.23 setFrequencyRegion【Set frequency region】	20

3.24 getFrequencyRegion【Query frequency region.】	21

3.25 addMask【add mask】	21

3.26 clearMask【clear mask】	21

3.27 unregisterCallback【Unregister callback】	22

3.28 getModuleFirmware【Get module firmware information】	22

3.29 setScanInterval【Set the inventory interval】	22

3.30 getScanInterval【Read Inventory Interval】	23

3.31 startRead【start inventory】	23

3.32 scanRfid【Take inventory over a period of time】	24

3.33 setInventoryParameter【configure inventory parameters】	24

3.34 getDeviceId【Gets the serial number】	24

3.35 setCustomRegion【Set custom bands】	25

3.36 getCustomRegion【Get a custom band】	25

3.37 setProfile【Set the link configuration information】	26

3.38 setRange【set the distance】	26

3.39 getRange【get the distance】	27

3.40 GetReaderType【get rfid mode】	27

3.41 getReaderTemperature【Query internal temperature】	27

3.42 startInventoryLed【Check ELD label】	27

3.43 stopInventoryLed【Stop querying ELD labels】	28

3.44 enableLog【Enable or disable logs】	28

3.45 inventoryOnce【One Query Inventory】	29

3.46 inventoryOnceMix【Single mixed inquiry】	30

3.47 writeTagEpc【Write the label EPC area】	31

3.48 writeTagEpcByTid【Write EPC area by tid】	31

Appendix 1: Return value error code table	33

Appendix 2: Frequency band Description	34

Appendix 3: RSSI parameter comparison table	36

- Common method usage process

- Instructions:

- Due to the relatively power consumption of the RFID module, it is recommended that the APP power off the RFID module when the APP is not using the RFID module or the APP is in the background (inactive state).

- The corresponding method is as follows (refer to the demo code for details) :

- 1> When using the RFID function or the APP is in the foreground:

- (This method is executed in about 2 seconds and requires logical processing)

- RFIDSDKManager.getInstance().init(InitListener);

- 2> When the RFID function is not used or the APP is in the background:

- RFIDSDKManager.getInstance().disConnect();

- RFIDSDKManager.getInstance().power(false);

  - initialization

- private void initRfid() { - RFIDManager.getInstance().init(new InitListener() { - @Override - public void onStatus(boolean status) { - if ( status ) { - Log.d(TAG, "initRfid() success."); - mRfidManager = USDKManager.getInstance().getRfidManager(); - }else { - Log.d(TAG, "initRfid fail."); - } - } - }); - }

  - Inventory

- After initializing RfidManager’s instance,you can make an inventory of cargo by using startInventory/startRead,and you can get the result from the callback method of OnInventoryTag,which is defined in the interface of IRfiCallbcak.

- steps：

- Start working：

- Rfidmanager.startInventory(int session);

- Implements the method onInventoryTag of the interface .

  - read tag

- You can get the tag’s information by using the readTag method of RfidManager.

- steps：

- Read tag：rfidmanager.readTag(epc,btMemBank,btWordAdd,btWordCnt,btAryPassWord)，the method will return what you want to get from the target tag.

  - write tag

- With rfidmanager's writeTag method.

- Steps:

- Write operation:

- //Write labels (any area and address)

- int ret =

- rfidmanager.writeTag(epc,btAryPassWord,btMemBank,btWordAdd,btAryData);

- //Write EPC area

- int ret =rfidmanager.writeTagEpc(epc,strPassword,strData);

- Method will return the result directly, 0 is success, other is failure.

- 1.5 get the power of output

- You can get current value of output by using

- RfidManager.getOutputPower,using the getOutputPower method to send command.

- Send the command of getting the power of output

- rfidmanager.getOutputPower()

- 1.6 set the power of output

- You can get current value of output by using the setOutputPower method ,using the getOutputPower method to send command.

- Set output power：rfidmanager.setOutputPower()

- 1.7 Parameter setting

- Parameter interface

- public class RfidParameter{

- public byte ComAddr;

- public int IvtType;

- public int Memory;

- public String Password;

- public int WordPtr;

- public int Length;

- public int Session;

- public int QValue;

- public int ScanTime;

- public int Antenna;

- public int Interval;

- public byte MaskMem;

- public byte MaskAdr[] = new byte[2];

- public byte MaskLen;

- public byte[]MaskData=new byte[32];

- }

| - Name | - type | - note |
| --- | --- | --- |
| - IvtType | - int | - 0-EPC query; 1- Hybrid query; 2-FastID. The default value is 0. |
| - Memory | - int | - Mixed search area, default 2 |
| - Password | - String | - Label access password, default 00000000 |
| - WordPtr | - int | - Mixed query start address, default 0 |
| - Length | - int | - Mixed query length, default 6 |
| - Session | - int | - 0 - 3. The default value is 0 |
| - QValue | - int | - 0 - 15. The default value is 4 |
| - ScanTime | - int | - The maximum response time of the query command is 20 (N x 100ms) by default. - For example, if 2000ms is set, 20 (20*100) is passed in. |
| - Interval | - int | - Query the interval between commands. The default is 0 |
| - MaskMem | - byte | - Mask area, default 2 |
| - MaskAdr | - byte[] | - Mask Start address. The default value is 000 |
| - MaskLen | - int | - Mask length. 0 indicates that the mask is disabled |
| - MaskData | - byte[] | - Mask data |

- Get current parameter：

- RfidParameter rfidParameter =  mRfidManager.getInventoryParameter();

- Set parameters：

- RfidParameter rfidParameter =  mRfidManager.getInventoryParameter();

- rfidParameter.Session = 0;

- rfidParameter.Interval = 0;

- rfidParameter.QValue = 6;

- .......

- mRfidManager.setInventoryParameter(rfidParameter);

- 1.8 Long range (with handle) device control scanning head light

- RFIDSDKManager.getInstance().enableScanHead(boolean isOpen);

- This method is for the long distance RFID device with a handle, control the button on the handle, whether to trigger the dock scanning code; Passing true triggers code scanning; Passing false does not trigger code scanning.

- Interface Callback

If you need to take inventory, you need to register the callback before inventory.

public interface IRfidCallback {

void onInventoryTag(String EPC,String TID,String RSSI);

void onInventoryTagEnd();

}

- The details about methods of the callback interface。

- note：there are some parameters do not work,they have a default value,it is necessary to focus on them in this version。

- onInventoryTag

| define | void onInventoryTag( String epc_data, String data, String rssi); |  |  |
| --- | --- | --- | --- |
| description | Get the result of making an inventory of cargo |  |  |
| parameter | name | type | note |
|  | epc | String | epc |
|  | data | String | If you choose mixed query mode, that data is exactly what you want.(tid，user) |
|  | rssi | String | rssi |

- onInventoryTagEnd

| define | void onInventoryTagEnd(); |  |  |
| --- | --- | --- | --- |
| description | When you call the method to stop inventory, the program executes this callback, which is the end. |  |  |
| parameters | name | type | note |

- RFID api

### 3.1 init【initialization】

| define | public void init(); |  |  |
| --- | --- | --- | --- |
| description | Initialize RFIDManager’s intance。 |  |  |
| parameters | name | type | note |
| return |  |  |  |
| Reference Code | None |  |  |

### 3.2 release【release resources】

| define | public void release(); |  |  |
| --- | --- | --- | --- |
| description | release resources |  |  |
| parameters | name | type | note |
| return |  |  |  |
| Reference Code |  |  |  |

### 3.3 disConnect【disconnect rfid module】

| define | public void disConnect(); |  |  |
| --- | --- | --- | --- |
| description | disconnect rfid module。 |  |  |
| parameters | name | type | note |
| return (void) |  |  |  |
| Reference Code | None |  |  |

### 3.4 isConnected【RFID connection status 】

| define | public boolean isConnected(); |  |  |
| --- | --- | --- | --- |
| description | Determine the connection status of the module |  |  |
| parameters | name | type | note |
| return (boolean) | success：true ；fail：false ； |  |  |
| Reference Code |  |  |  |

- 3.5 registerCallback【register CallBack】

| define | public void registerCallback(IRfidCallback cb); |  |  |
| --- | --- | --- | --- |
| description | Unregister CallBack。 |  |  |
| parameters | name | type | note |
|  | cb | IRfidCallback | Callback |
| return (void) |  |  |  |
| Reference Code |  |  |  |

### 3.6  startInventory【User define session and target inventory】

| define | public int startInventory( int session); |  |  |
| --- | --- | --- | --- |
| description | User define session and target inventory |  |  |
| parameters | name | type | note |
|  | session | int | Desired session ID. Values(0;1;2;3) |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

### 3.7 stopInventory【stop tag inventory】

| define | public int stopInventory(); |  |  |
| --- | --- | --- | --- |
| description | stop tag inventory |  |  |
| parameters | name | type | note |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.8 readTag【read tag】

| define | public int readTag( String EPC, int btMemBank, int btWordAdd, int btWordCnt, String strPassWord); |  |  |
| --- | --- | --- | --- |
| description | Read Tag. Attention: If two tags have the same EPC, but different read data, then these two tags are considered different tags. |  |  |
| parameters | name | type | note |
|  | epc | String | EPC value |
|  | btMemBank | int | Tag memory bank(0:RESERVED, 1:EPC, 2:TID, 3:USER) |
|  | btWordAdd | int | Read start address,please see the tag's spec for more information. |
|  | btWordCnt | int | Read data length,Data length in WORD(16bits) unit. Please see the tag's spec for more information. |
|  | strPassWord | String | Access password,4 bytes. |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.9 readDataByTid【Tag data is read via TID】

| define | public String readDataByTid(String TIDStr, int Mem, int WordPtr, int Num, String Password) |  |  |
| --- | --- | --- | --- |
| description | Tag data is read via TID |  |  |
| parameters | name | Type | note |
|  | TIDStr | String | tid |
|  | Mem | int | The region that needs to be read |
|  | WordPtr | int | The start address of the data to be read |
|  | Num | int | Length of data |
|  | Password | String | Access password |
| return (String) | - Null:fail |  |  |
| Reference Code |  |  |  |

### 3.10 inventorySingle【single tag query】

| define | public int inventorySingle( ) |  |  |
| --- | --- | --- | --- |
| description | Only one label is read per call. |  |  |
| parameters | name | type | note |
| return | The onInventoryTag() callback returns the corresponding result. |  |  |
| Reference Code | 。 |  |  |

### 3.11 writeTag【Write Tag】

| define | public int writeTag(String epc，String strPassWord, int btMemBank, int btWordAdd,int btWordCnt,String wData); |  |  |
| --- | --- | --- | --- |
| description | Write Tag |  |  |
| parameters | name | type | note |
|  | epc | String | epc |
|  | strPassWord | String | Access password, 4 bytes. |
|  | btMemBank | int | Tag memory bank(0:RESERVED, 1:EPC, 2:TID, 3:USER) |
|  | btWordAdd | int | Write start address,WORD(16 bits). When write EPC area, notice that EPC starts from address 02, the first two 2 words are for PC+CRC. |
|  | btWordCnt | int | WORD(16 bits), please see the tag's spec for more information. |
|  | wData | String | Write data |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

### 3.12 writeTagByTid【Write data to each storage area through tid】

| define3 | public int writeTagByTid(String TIDStr,int Mem, int WordPtr,String Password,String wdata); |  |  |
| --- | --- | --- | --- |
| description | Write data to each memory area. |  |  |
| parameters | name | type | note |
|  | TIDStr | String | The tid number of the label, a hexadecimal string, the length must be an integer multiple of 4 |
|  | Mem | int | - memory area to be written, - 0- Password area, the first 3 words are the destruction password, the last 3 words are the access password - 1-EPC area - 2-TID area - User area |
|  | WordPtr | int | Write start word address |
|  | Password | String | The access password of the label, in hexadecimal string format。 |
|  | WData | String | The data to be written, a hexadecimal string, the length must be an integer multiple of 4 |
| return (int) | Success:0,fail:-1 |  |  |
| Reference Code |  |  |  |

### 3.13 writeEpc【write epc number】

| define1 | public int WriteEPC(int epclen,String epc,String Password); |  |  |
| --- | --- | --- | --- |
| description | Randomly rewrite the EPC number of a label by broadcasting |  |  |
| parameters | name | type | note |
|  | epclen | int | length of epc |
|  | epc | String | Input, the EPC number of the label |
|  | Password | String | Input, the access code for the tag, 4 bytes |
| return (int) | successreturn0。 |  |  |
| Reference Code |  |  |  |

- 3.14 killTag【Kill Tag】

| define | public int killTag(String epc,String strPassWord); |  |  |
| --- | --- | --- | --- |
| description | Kill Tag |  |  |
| parameters | name | type | note |
|  | epc | String | epc value |
|  | strPassWord | String | Kill password,4 bytes |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

### 3.15 KillbyTID【The tag is destroyed by TID】

| define | public int KillbyTID(int tidlen, String tid, String Password); |  |  |
| --- | --- | --- | --- |
| description | The tag is destroyed by TID。 |  |  |
| parameters | name | type | note |
|  | tidlen | int | TID length |
|  | tid | String | Tid value |
|  | Password | String | Kill password,4 bytes. |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.16 lockTag【Lock Tag】

| define | public int lockTag( String epc, String strPassWord, int btMemBank, int btLockType); |  |  |
| --- | --- | --- | --- |
| description | Lock Tag |  |  |
| parameters | name | type | note |
|  | epc | String | epc value |
|  | strPassWord | String | Tag access password, 4 bytes |
|  | btMemBank | int | 1 byte, 0 - Control Kill Password read/write protection Settings. 1 - Control access password read/write protection Settings. 2 -- Controls EPC memory read/write protection Settings. 3 - Control TID memory read/write protection Settings. 4 - Controls user memory read/write protection Settings. Other values are reserved, and if the outgoing reader receives other values, it returns a message that the parameter is incorrect. |
|  | btLockType | int | 1 byte, When Select is 0 or 1, the SetProtect value represents the following: 0 - Set to read-write 1 - Set to always read and write 2 - Set to read-write with password 3 - Set to never read or write When Select is 2, 3, or 4, the SetProtect value represents the following: 0 - Set to writable 1 - Set to always writable 2 - Set to writable with password 3 - Set to never write. |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

### 3.17 LockbyTID【The tag is locked using the TID】

| define | public int lockTag(int tidlen, String tid,int select, int setprotect,String Password); |  |  |
| --- | --- | --- | --- |
| description | The tag is locked using the TID。 |  |  |
| parameters | name | type | note |
|  | tidlen | int | TID length of the tag |
|  | tid | String | Enter the TID of the tag. |
|  | select | int | 1 byte, 0 - Control Kill Password read/write protection Settings. 1 - Control access password read/write protection Settings. 2 -- Controls EPC memory read/write protection Settings. 3 - Control TID memory read/write protection Settings. 4 - Controls user memory read/write protection Settings. Other values are reserved, and if the outgoing reader receives other values, it returns a message that the parameter is incorrect. |
|  | setprotect | int | 1 byte, When Select is 0 or 1, the SetProtect value represents the following: 0 - Set to read-write 1 - Set to always read and write 2 - Set to read-write with password 3 - Set to never read or write When Select is 2, 3, or 4, the SetProtect value represents the following: 0 - Set to writable 1 - Set to always writable 2 - Set to writable with password 3 - Set to never write. |
|  | Password | String | Access password, 4 bytes. |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.18 setQueryMode【config query model】

| define | public void setQueryMode(int mode) |  |  |
| --- | --- | --- | --- |
| description | Config query model |  |  |
| parameters | name | type | note |
|  | Mode | Int | 0：epc 1：epc+tid 2：epc+user 3：fasttid |
| return (void) |  |  |  |
| Reference Code | none |  |  |

- 3.19 getQueryMode【get query mode】

| define | public int getQueryMode() |  |  |
| --- | --- | --- | --- |
| description | Get the query mode |  |  |
| parameters | name | type | note |
| return (int) | 0：epc - 1：epc+tid - 2：epc+user - 3：fasttid |  |  |
| Reference Code | none |  |  |

### 3.20 getFirmwareVersion【Get Reader Firmware Version.】

| define | public String getFirmwareVersion(); |  |  |
| --- | --- | --- | --- |
| description | Get Reader Firmware Version. |  |  |
| parameters | name | type | note |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.21 setOutputPower【Set output power】

| define | public int setOutputPower(int btOutputPower); |  |  |
| --- | --- | --- | --- |
| description | Set the reader power |  |  |
| parameters | name | type | note |
|  | btOutputPower | int | Rf output power: Long power range: 0-33 Short-range version power range: 0-26 |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.22 getOutputPower【Query output power】

| define | public int getOutputPower(); |  |  |
| --- | --- | --- | --- |
| description | Query output power。 |  |  |
| parameters | name | type | note |
| return (int) | Return output power. |  |  |
| Reference Code |  |  |  |

- 3.23 setFrequencyRegion【Set frequency region】

| define | public int setFrequencyRegion( int btRegion, int btStartRegion, int btEndRegion); |  |  |
| --- | --- | --- | --- |
| description | Set frequency region(system default frequencies). |  |  |
| parameters | name | type | note |
|  | btRegion | int | One byte, spectrum band. 1 - "Chinese band2"; 2 - "US band"; 3 - "Korean band"; 4 - "EU band"; 8 - "Chinese band1"; For details : (Appendix 2: Frequency band Description). |
|  | btStartRegion | int | Start frequency of the spectrum. |
|  | btEndRegion | int | End frequency of the spectrum,Setup the range of the RF output spectrum. The rules are: 1,Start frequency and end frequency should be in the range of the specified regulation. 2,Start frequency should be equal or lower than end frequency. 3, End frequency equals start frequency means use single frequency point. |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

### 3.24 getFrequencyRegion【Query frequency region.】

| define | public RfidDate getFrequencyRegion(); |  |  |
| --- | --- | --- | --- |
| description | Query frequency region. |  |  |
| parameters | name | type | note |
| return (RfidDate ) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.25 addMask【add mask】

| define | public void addMask(int mem,int startAddress,int len,String data) |  |  |
| --- | --- | --- | --- |
| description |  |  |  |
| parameters | name | Type | note |
|  | mem | int | Filter area |
|  | startAddress | int | Start address |
|  | len | int | Length of data |
|  | data | String | Filter data |
| return (void) |  |  |  |
| Reference Code |  |  |  |

- 3.26 clearMask【clear mask】

| define | public void clearMask(); |  |  |
| --- | --- | --- | --- |
| description | Clear so the mask |  |  |
| parameters | name | Type | note |
| return (void) |  |  |  |
| Reference Code |  |  |  |

### 3.27 unregisterCallback【Unregister callback】

| define | public void unregisterCallback(IRfidCallback cb); |  |  |
| --- | --- | --- | --- |
| description | Unregister callback |  |  |
| parameters | name | type | note |
|  | cb | IRfidCallback | Callback |
| return (void) |  |  |  |
| Reference Code |  |  |  |

### 3.28 getModuleFirmware【Get module firmware information】

| define | public String getModuleFirmware(); |  |  |
| --- | --- | --- | --- |
| description | Get module firmware information |  |  |
| parameters | name | type | note |
| return (String) | module firmware information |  |  |
| Reference Code |  |  |  |

### 3.29 setScanInterval【Set the inventory interval】

| define | public int setScanInterval( int interval) |  |  |
| --- | --- | --- | --- |
| description | Set the query parameters to be used when inventory is enabled |  |  |
| parameters | name | type | note |
|  | interval | int | The parameter ranges from 0 to 6. Corresponding to: 0-0ms 1-10ms 2-20ms 3-30ms 4-40ms 5-50ms 6-60ms That is, interval*10ms is the actual interval, in milliseconds. |
| return (void) | 0:success -1:fail |  |  |
| Reference Code |  |  |  |

### 3.30 getScanInterval【Read Inventory Interval】

| define | public int getScanInterval() |  |  |
| --- | --- | --- | --- |
| description | Read the query used when inventory is enabled parameters |  |  |
| parameters | name | type | note |
| return (int) | Return 0-6, the obtained interval time value *10ms to get the actual interval time, in ms; Return -1, calling the method failed |  |  |
| Reference Code |  |  |  |

### 3.31 startRead【start inventory】

| define | public int startRead() |  |  |
| --- | --- | --- | --- |
| description | You can configure your own inventory parameters before calling the method |  |  |
| parameters | name | type | note |
| return (int) | - 0:success - -1:fail |  |  |
| Reference Code | none |  |  |

### 3.32 scanRfid【Take inventory over a period of time】

| define | public void scanRfid() |  |  |
| --- | --- | --- | --- |
| description | Take inventory over a period of time |  |  |
| parameters | name | type | note |
| return (void) |  |  |  |
| Reference Code | none |  |  |

### 3.33 setInventoryParameter【configure inventory parameters】

| define | public void setInventoryParameter(Rfidparameter parameter) |  |  |
| --- | --- | --- | --- |
| description | configure inventory parameters |  |  |
| parameters | name | type | note |
| return (void) |  |  |  |
| Reference Code | none |  |  |

### 3.34 getDeviceId【Gets the serial number】

| define | public String getDeviceId() |  |  |
| --- | --- | --- | --- |
| description | Gets the serial number |  |  |
| parameters | name | Type | note |
| return (String ) | - the serial number |  |  |
| Reference Code | none |  |  |

### 3.35 setCustomRegion【Set custom bands】

| define | public int setCustomRegion(int flags, int FreSpace, int FreNum, int StartFre); |  |  |
| --- | --- | --- | --- |
| description | Set custom bands |  |  |
| parameters | name | Type | note |
|  | flags | int | Whether to save the setting information ID when powered 0：save 1：clear |
|  | int | FreSpace | Band spacing |
|  | int | FreNum | Number of frequency points |
|  | int | StartFre | Starting band |
| return (int) | - 0:success |  |  |
| Reference Code |  |  |  |

### 3.36 getCustomRegion【Get a custom band】

| define | public CustomRegionBean getCustomRegion(); |  |  |
| --- | --- | --- | --- |
| description | Get a custom band information |  |  |
| parameters | name | Type | note |
| return (CustomRegionBean ) | - Null:fail - public class CustomRegionBean { - public int [] band=new int[1];// - public int [] FreSpace=new int[1];// - public int [] FreNum=new int[1];// - public int [] StartFre=new int[1];//d - public CustomRegionBean(){} - } |  |  |
| Reference Code |  |  |  |

### 3.37 setProfile【Set the link configuration information】

| define | public int setProfile(byte param); |  |  |
| --- | --- | --- | --- |
| description | Set the link configuration information |  |  |
| parameters | name | Type | note |
|  | param | int | Generally, the optional parameters of this parameter need to be provided according to the device information. If the type of reader is（0x21,0x23,0x28,0x36,0x37） parm： 0:40K, FM0,25us 1:250K,M4, 25us 2:300K,M4, 25us 3:400K,FM0,6.25us Other 11:640K,FM0,7.5u 1:640K, M2,7.5us 15:640K, M4,7.5us 12:320K, M2, 15us 3:320K, M2, 20us 5:320K, M4, 20us 7:250K, M4, 20us 13:160K, M8, 20us |
| Void |  |  |  |
| Reference Code |  |  |  |

### 3.38 setRange【set the distance】

| define | public int setRange(int range); |  |  |
| --- | --- | --- | --- |
| description | Set the distance at which the label can be identified |  |  |
| parameters | name | Type | note |
|  | Int | range | 0~100 |
| Int | - 0:success; |  |  |
| Reference Code |  |  |  |

### 3.39 getRange【get the distance】

| define | public int getRange(); |  |  |
| --- | --- | --- | --- |
| description | get the distance at which the label can be identified |  |  |
| parameters | name | Type | note |
| Int | - -1:fail; - Other:the distance |  |  |
| Reference Code |  |  |  |

- 3.40 GetReaderType【get rfid mode】

| define | public int GetReaderType() |  |  |
| --- | --- | --- | --- |
| description | Get rfid mode |  |  |
| parameters | name | type | note |
| return (int) | - 80：the mode of short distance - others：the mode of long distance mode |  |  |
| Reference Code | none |  |  |

- 3.41 getReaderTemperature【Query internal temperature】

| define | public RfidDate getReaderTemperature(); |  |  |
| --- | --- | --- | --- |
| description | Query internal temperature。 |  |  |
| parameters | name | type | note |
| return (int) |  |  |  |
| Reference Code |  |  |  |

### 3.42 startInventoryLed【Check ELD label】

| define | public int startInventoryLed(int manufacturers, List<String> epcs); |  |  |
| --- | --- | --- | --- |
| description | Check ELD label |  |  |
| parameters | name | type | note |
|  | manufacturers | int | manufacturer |
|  | epcs | List<String> | epc to be queried |
| return (int) | Success: 0; Failure: other; |  |  |

### 3.43 stopInventoryLed【Stop querying ELD labels】

| define | public void stopInventoryLed(); |  |  |
| --- | --- | --- | --- |
| description | Stop querying ELD labels |  |  |
| parameters | name | type | note |
| return (void) |  |  |  |

### 3.44 enableLog【Enable or disable logs】

| define | public void enableLog(boolean enable); |  |  |
| --- | --- | --- | --- |
| description | Enable or disable logs。 |  |  |
| parameters | name | type | note |
|  | enable | boolean | true: Enable logs. false: disable logs |
| return (void) |  |  |  |

- 3.45 inventoryOnce【One Query Inventory】

| define | public int InventoryOnce(int session, int qvalue, int tidAddr, int tidLen, int target, int scantime) |  |  |
| --- | --- | --- | --- |
| description | This command is used to search for nearby labels. The query is performed only once and multiple labels are returned. The result is called back in the onInventoryTag() method, and the onInventoryTagEnd() method is called back at the end of a single session. |  |  |
| parameters | name | type | note |
|  | session | int | 1 byte: Session value used when querying EPC label. 0: The Session uses S0. 1: Session uses S1; 2: Session uses S2; 3: The Session uses S3. S0 is recommended for single sheets or a small number of labels. |
|  | qvalue | int | The initial Q value used when querying EPC labels ranges from 0 to 15.(0 or 6 is recommended) |
|  | tidAddr | int | Query the start word address in the TID area |
|  | tidLen | int | Query the number of characters in the TID area. The value ranges from 0 to 15. If the value is 0, query the EPC number of the label. (If the length is 0, the TID area is not obtained.) |
|  | target | int | 1 byte to query the Target value of the tag. 0 - Target A; 1 - Target B. |
|  | scantime | int | 1 byte: Maximum time for querying the command. The value ranges from 3 to 255(*100ms) (2 seconds is recommended). |
| return (int) | Success: 0; Failure: other. |  |  |
| Reference Code | inventoryOnce((byte)1,(byte)4,(byte)0,(byte)0,(byte)0,(byte)50); |  |  |

- 3.46 inventoryOnceMix【Single mixed inquiry】

| define | public int inventoryOnceMix(int session, int qvalue, int target, int scantime, int readMem, byte[] ReadAdr, int readLen, String pwd) |  |  |
| --- | --- | --- | --- |
| description | This command is used to search for nearby labels. If you query the EPC+ storage area, multiple labels are returned. The result is called back in the onInventoryTag() method, and the onInventoryTagEnd() method is called back at the end of a single session. |  |  |
| parameters | Name | type | note |
|  | session | int | 1 byte: Session value used when querying EPC label. 0: The Session uses S0. 1: Session uses S1; 2: Session uses S2; 3: The Session uses S3. S0 is recommended for single sheets or a small number of labels. |
|  | qvalue | int | The initial Q value used when querying EPC labels ranges from 0 to 15.(0 or 6 is recommended) |
|  | target | int | 1 byte to query the Target value of the tag. 0 - Target A; 1 - Target B. |
|  | scantime | int | 1 byte: Maximum time for querying the command. The value ranges from 3 to 255(*100ms) (2 seconds is recommended). |
|  | readMem | int | Label storage area (0: RESERVED, 1: EPC, 2: dar, 3: the USER). |
|  | readAdr | byte[] | Fixed 2 bytes, the first byte is the high byte, the second byte is the low byte, such as 6 Length: new byte[]{0, 6} |
|  | readLen | int | Queries the data length of the specified storage area. The length must be greater than 0. |
|  | pwd | String | Tag access password, 4 bytes |
| return (int) | Success: 0; Failure: other. |  |  |
| Reference Code | Read EPC+TID: byte[] readAdr = new byte[]{ 0x00 , 0x06 };//6 characters in length InventoryOnceMix(1,4,0,50,2,readAdr, 6, "00000000"); |  |  |

### 3.47 writeTagEpc【Write the label EPC area】

| define | public int writeTagEpc(String selectEpc, String password, String writeData ); |  |  |
| --- | --- | --- | --- |
| description | Modify the EPC value of the label and change the original EPC value to the written EPC value, that is, change the EPC number of the label to writeData. |  |  |
| parameters | Name | type | note |
|  | selectEpc | String | epc of the selected label |
|  | password | String | Enter password 4 bytes |
|  | writeData | String | Data to be written (hexadecimal, a multiple of length 4) |
| return (int) | Return 0 on success. |  |  |
| Reference Code |  |  |  |

### 3.48 writeTagEpcByTid【Write EPC area by tid】

| define | public int writeTagByTid(String TIDStr, String password, String writeData ); |  |  |
| --- | --- | --- | --- |
| description | Modify the EPC value of the tag. Change the current EPC value of the tag to the written EPC value through the TID value, that is, change the EPC number of the tag to writeData. |  |  |
| parameters | Name | type | note |
|  | TIDStr | String | tid number of the label |
|  | Password | String | Label access password. The value is in hexadecimal string format. |
|  | WData | String | The value is a hexadecimal character string. The length must be an integer multiple of 4. |
| return (int) | Return 0 on success. |  |  |
| Reference Code |  |  |  |

### 3.49 findEpc【Find Tag】

| define | public Tag6C findEpc(String selectEpc); |  |  |
| --- | --- | --- | --- |
| description | Finds and locates the specified label |  |  |
| parameters | Name | type | note |
|  | selectEpc | String | epc number of the label |
| return (Tag6C ) | Successful return Tag6C is not empty. The rssi field of Tag6C is used to judge the strength of the signal, so as to judge whether it is close to the label and make positioning judgment. |  |  |
| Reference Code |  |  |  |

# Appendix 1: Return value error code table

| Error code | description |
| --- | --- |
| 0 | Execution succeeded. |
| 1 | The electronic tag was not found. |
| 2 | The power supply is insufficient. The label cannot be written to the storage system. |
| 3 | A PC value that is out of memory or unsupported, indicating that the storage location does not exist or is marked The PC value is not supported. |
| 4 | Memory lock, storage location lock permanently locked, and cannot be written. |
| 5 | The access password is incorrect. |
| 9 | Destroy password errors. |
| 10 | The destruction password cannot be all zeros. |
| 11 | The command is not supported for electronic tags. |
| 12 | For this command, the access password cannot be 0. |
| 13 | The electronic tag has been set with read protection and cannot be set again. |
| 14 | The electronic tag is not set to read protection and does not need to be unlocked. |
| 15 | Non-specific error, the tag does not support specific error codes. |
| 16 | Byte space is locked and write failed. |
| 17 | Cannot be locked. |
| 18 | Already locked and cannot be locked again. |
| 19 | The parameter failed to be saved, but the value set was valid before the read-write module was powered off. |
| 20 | Cannot be adjusted. |
| 248 | Antenna detection error |
| 249 | An error occurred in the execution of the command. |
| 250 | There are electronic tags, but the communication is not smooth and it cannot be operated. |
| 251 | No electronic tags are operable. |
| 252 | The electronic tag returns an error code. |
| 253 | The command length is incorrect. |
| 254 | Illegal orders. |
| 255 | Parameter error. |
| 48 | Communication error. |
| -1 | Uninitialized. |

# Appendix 2: Frequency band Description

EU Frequency band（865~868MHz）

| Band name | Number of frequency points | Channel intervalkHz | Frequency point formula（MHz） | Area code（hexadecimal） | range（MHZ） |
| --- | --- | --- | --- | --- | --- |
| EU | 15 | 200 | 865.1 + N*0.2(MHz) Among them N∈[0, 14] | 4 | 865.1-867.9 |
| Ukraine | 7 | 100 | 868.0 + N*0.1(MHz) Among them N∈[0, 6] | 6 | 868.0-868.6 |
| Chinese 1 | 20 | 250 | 840.125 + N * 0.25 (MHz) Among them N∈[0, 19] | 8 | 840.125-844.875 |
| EU3 | 4 | 600 | 865.7 + N * 0.6(MHz) Among them N∈[0, 3] | 9 | 865.7-867.5 |

Examples of setting these region-specific frequency bands:

For example EU：（It corresponds to parameters 0 to 14 from top to bottom）

865.1  0

865.3  1

865.5  2

865.7  3

865.9  4

866.1  5

866.3  6

866.5  7

866.7  8

866.9  9

867.1  10

867.3  11

867.5  12

867.7  13

867.9  14

If you need to set EU: 865.3-867.5

mRfidManager.setFrequencyRegion(4, 2, 12);

Obtain information about the frequency band set in the preceding method.

RfidDate frequencyRegion = mRfidManager.getFrequencyRegion();

frequencyRegion.btRegion: range（4）

frequencyRegion.btFrequencyStart: initial frequency point（2）

frequencyRegion.btFrequencyEnd: cut-off frequency point（12）

# Appendix 3: RSSI parameter comparison table

| RSSI | Signal Strength | RSSI | Signal Strength |
| --- | --- | --- | --- |
| 98(0x62) | -31dBm | 64(0x40) | -66dBm |
| 97(0x61) | -32dBm | 63(0x3F) | -67dBm |
| 96(0x60) | -33dBm | 62(0x3E) | -68dBm |
| 95(0x5F) | -34dBm | 61(0x3D) | -69dBm |
| 94(0x5E) | -35dBm | 60(0x3C) | -70dBm |
| 93(0x5D) | -36dBm | 59(0x3B) | -71dBm |
| 92(0x5C) | -37dBm | 58(0x3A) | -72dBm |
| 91(0x5B) | -38dBm | 57(0x39) | -73dBm |
| 90(0x5A) | -39dBm | 56(0x38) | -74dBm |
| 89(0x59) | -41dBm | 55(0x37) | -75dBm |
| 88(0x58) | -42dBm | 54(0x36) | -76dBm |
| 87(0x57) | -43dBm | 53(0x35) | -77dBm |
| 86(0x56) | -44dBm | 52(0x34) | -78dBm |
| 85(0x55) | -45dBm | 51(0x33) | -79dBm |
| 84(0x54) | -46dBm | 50(0x32) | -80dBm |
| 83(0x53) | -47dBm | 49(0x31) | -81dBm |
| 82(0x52) | -48dBm | 48(0x30) | -82dBm |
| 81(0x51) | -49dBm | 47(0x2F) | -83dBm |
| 80(0x50) | -50dBm | 46(0x2E) | -84dBm |
| 79(0x4F) | -51dBm | 45(0x2D) | -85dBm |
| 78(0x4E) | -52dBm | 44(0x2C) | -86dBm |
| 77(0x4D) | -53dBm | 43(0x2B) | -87dBm |
| 76(0x4C) | -54dBm | 42(0x2A) | -88dBm |
| 75(0x4B) | -55dBm | 41(0x29) | -89dBm |
| 74(0x4A) | -56dBm | 40(0x28) | -90dBm |
| 73(0x49) | -57dBm | 39(0x27) | -91dBm |
| 72(0x48) | -58dBm | 38(0x26) | -92dBm |
| 71(0x47) | -59dBm | 37(0x25) | -93dBm |
| 70(0x46) | -60dBm | 36(0x24) | -94dBm |
| 69(0x45) | -61dBm | 35(0x23) | -95dBm |
| 68(0x44) | -62dBm | 34(0x22) | -96dBm |
| 67(0x43) | -63dBm | 33(0x21) | -97dBm |
| 66(0x42) | -64dBm | 32(0x20) | -98dBm |
| 65(0x41) | -55dBm | 31(0x1F) | -99dBm |